Geological Magazine

Detrital zircon provenance of north Gondwana Palaeozoic sandstones from Saudi Arabia

Guido Meinhold1,2, Alexander Bassis3,4, Matthias Hinderer3, Anna Lewin3 and Jasper Berndt5

1School of Geography, Geology and the Environment, Keele University, Keele, Staffordshire, ST5 5BG, UK; 2Abteilung Sedimentologie/Umweltgeologie, Geowissenschaftliches Zentrum Göttingen, Universität Göttingen, Goldschmidtstraße 3, 37077 Göttingen, Germany; 3Institut für Angewandte Geowissenschaften, Technische Universität Darmstadt, Schnittspahnstrasse 9, 64287 Darmstadt, Germany; 4Eurofins water&waste GmbH, Eumigweg 7, 2351 Wiener Neudorf, Austria and 5Institut für Mineralogie, Westfälische Wilhelms-Universität Münster, Corrensstraße 24, 48149 Münster, Germany

Author for correspondence: Guido Meinhold, Email: g.meinhold@keele.ac.uk

Supplementary Material

Table S1. Laser ablation ICP–MS operation parameters.

| Laboratory | Institut für Mineralogie, Westfälische Wilhelms-Universität Münster |
| --- | --- |
| Laser ablation system | Photon Machines, Analyte G2, Excimer laser |
| Ablation cell | HelEx 2-volume cell |
| Wavelength | 193 nm |
| Pulse width | 4 ns |
| Fluence | 3–4 J/cm² |
| Repetition rate | 10 Hz |
| Ablation time | 37 s |
| Spot size | 35 µm (most samples), 25 µm (a few samples) |
| Sampling mode | Static |
| Carrier gas | He in the cell, Ar sampling and cooling gas |
| Carrier gas flow | 0.9 L/min for mass-flow controller 1; 0.4 L/min for mass-flow controller 2 |
| ICP–MS instrument | ThermoFisher Element 2, single-collector ICP–MS |
| Radio frequency power | 1250 W |
| Sample and cooling gas flow | 1 L/min; 16 L/min |
| Detection system | SEM |
| Masses measured | 202, 204, 206, 207, 238 |
| Settling time | 1 ms/amu |
| Sample time | 40 ms (202, 204, 207), 10 ms (206), 4 ms (238) |
| Sweep time | 160 ms (202, 204, 207), 40 ms (206), 10 ms (238) |
| Integration time | 0.56 ms |
| Number of runs | 91 |
| Background time | 12 s |