Assessment of coastal dune vulnerability to natural and anthropogenic disturbances along the Gulf of Mexico

M. LUISA MARTÍNEZ, JUAN B. GALLEGO-FERNÁNDEZ, JOSÉ G. GARCÍA-FRANCO, CORAL MOCTEZUMA, CLAUDIA D. JIMÉNEZ

Environmental Conservation 33(2): 109-117

Appendix 1 Variables considered in the dune vulnerability classification assessment, according to Williams et al. (2001) and García-Mora et al. (2001).

| Variables considered |  | Value range |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Geomorphological condition of the dune system (GC) |  |  |  |  |  |  |
|  | 1. Length of homogeneous active dune system (km) |  0 >20 |  1 >10 |  2 >5 |  3 >1 |  4 >0.1 |
|  | 2. Width of dynamic dune system (km) |  0 >2 |  1 >1 |  2 >0.5 |  3 >0.1 |  4 <0.1 |
|  | 3. Width of front dune as % of active dune system |  0 <5% |  1 <25% |  2 <50% |  3 <75% |  4 >75% |
|  | 4. Average height of secondary dunes (m) |  0 >25 |  1 >10 |  2 >5 |  3 >1 |  4 <1 |
|  | 5. Average height of frontal dunes (m) |  0 >25 |  1 >15 |  2 >10 |  3 >5 |  4 <5 |
|  | 5a. If any ridges, number of major ridges |  0 >10 |  1 >4 |  2 >2 |  3 2 |  4 1 |
|  | 5b. If plastered to slope, slope steepness |  0 Moderate |  |  2 Gentle |  |  4 Steep |
|  | 5c. If perched on cliff, cliff height (m) |  0 <2 |  |  2 2–5 |  |  4 >5 |
|  | 6. Relative area of wet slacks (%) |  0 Moderate |  |  2 Small |  |  4 None |
|  | 7. Degree of dune system fragmentation |  0 Low |  |  2 Medium |  |  4 High |
|  | 8. Particle size of the frontal dune (phy-sizes) |  0 <1 |  |  2 1–2 |  |  4 >2 |
|  | Total score/per cent |  |  |  |  |  |
| 2. Marine influence (MI) |  |  |  |  |  |  |
|  | 1. Orthogonal fetch (km) |  0 <25 |  1 <100 |  2 <250 |  3 >500 |  4 >1000 |
|  | 2. Berm slope (degrees) |  0 Moderate |  |  2 Gentle |  |  4 Steep |
|  | 3. Width of intertidal zone (km) |  0 >0.5 |  1 >0.2 |  2 >0.1 |  3 >0.05 |  4 <0.05 |
|  | 4. Tidal range (m) |  0 >2 |  |  2 2–4 |  |  4 >4 |
|  | 5. Coastal orientation to wave direction (degrees) |  0 10–450 |  |  2 0–100 |  |  4 0 |
|  | 6. Width of the zone between HWSM and dune face (m) |  0 >75 |  1 <75 |  2 <25 |  3 <10 |  4 0 |
|  | 7. Breaches in the frontal dune due to washover, relative to total area |  0 0% |  1 <5% |  2 <25% |  3 <50% |  4 >50% |
|  | 8. % frontal dune cliff by the sea or with only ephemeral dunes as % of dune height |  0 0% |  1 <25% |  2 >25% |  3 >50% |  4 >75% |
|  | 9. Particle size of the beach (phy-sizes) |  0 <1 |  |  2 1–2 |  |  4 >2 |
|  | Total score/per cent |  |  |  |  |  |
| 3. Aeolian influence (AI) |  |  |  |  |  |  |
|  | 1. Sand supply input |  0 Moderate |  |  2 Gentle |  |  4 Steep |
|  | 2. % cover of embryo dunes along the seaward edge |  0 >50% |  1 >25% |  2 >5% |  3 <5% |  4 0% |
|  | 3. Blowouts: % of the system |  0 <5% |  1 <10% |  2 <25% |  3 <50% |  4 >50% |
|  | 4. Aeolian breaches in seaward face not induced by trampling: % of the system |  0 <5% |  1 <10% |  2 <25% |  3 <50% |  4 >50% |
|  | 4a. If breaches, depth as % of dune height |  0 <5% |  1 <10% |  2 <25% |  3 <50% |  4 >50% |
|  | 5. Natural litter driftcover as % surface |  0 0% |  1 <5% |  2 >5% |  3 >25% |  4 >50% |
|  | 6. Pebble cover as % surface |  0 0% |  1 <5% |  2 >5% |  3 >25% |  4 >50% |
|  | 7. Shell cover as % surface on upper beach |  0 0% |  1 <5% |  2 >5% |  3 >25% |  4 >50% |
|  | 8. % seaward dune vegetated |  0 >90% |  1 >60% |  2 >30% |  3 >10% |  4 <10% |
|  | 9. % of the system unvegetated |  0 <10% |  1 >10% |  2 >20% |  3 >40% |  4 >75% |
|  | Total score/per cent |  |  |  |  |  |
| 4. Vegetation condition (VC) |  |  |  |  |  |  |
|  | 1. % cover of type III plants on the beach |  0 >50% |  1 >25% |  2 >15% |  3 >5% |  4 <5% |
|  | 2. % cover type III plants on the seaside of the frontal dune |  0 >90% |  1 >60% |  2 >30% |  3 >15% |  4 <15% |
|  | 3. Relative proportion of Type II plants on the seaside of the frontal dune (% cover) |  0 <5% |  1 <15% |  2 <30% |  3 <60% |  4 >60% |
|  | 4. Relative proportion of Type I plants on the seaside of the frontal dune (% cover) |  0 <1% |  1 >1% |  2 >5% |  3 >10% |  4 >30% |
|  | 5. Relative proportion of exotic species on the seaside of the frontal dune (% cover) |  0 0% |  1 <1% |  2 <5% |  3 <15% |  4 >15% |
|  | 6. Relative proportion of Type II & III plants 100 m inland from the dune front |  0 >75 |  1 >50 |  2 >25 |  3 >10 |  4 <10 |
|  | 7. Relative proportion of vigorous plants or plants with normal vitality on the seaside of the frontal dune (%) |  0 >75% |  1 >50% |  2 >25% |  3 >10% |  4 <10% |
|  | 8. Relative cover (%) of exposed roots on the seaside of the frontal dune |  0 <5% |  1 >5% |  2 >15% |  3 >25% |  4 >50% |
|  | 9. Relative proportion (%) of plants with obvious physical damage |  0 <5 |  1 >5 |  2 >15 |  3 >25 |  4 >50 |
|  | 10. % vegetation removal seaward of the frontal dune due to human disturbance |  0 <10% |  1 >10% |  2 >25% |  3 >50% |  4 >75% |
|  | Total score/per cent |  |  |  |  |  |
| 5. Human effect (HE) |  |  |  |  |  |  |
|  | 1. Visitor pressure |  0 Low |  |  2 Moderate |  |  4 High |
|  | 2. Visitor frequency |  0 Low |  |  2 Moderate |  |  4 High |
|  | 3. Access difficulty |  0 High |  |  2 Moderate |  |  4 Low |
|  | 4. On dune driving |  0 None |  |  2 Some |  |  4 Much |
|  | 5. On beach driving |  0 None |  |  2 Some |  |  4 Much |
|  | 6. Horseback riding |  0 None |  |  2 Some |  |  4 Much |
|  | 7. Path network as % of frontal dune |  0 0% |  1 <5% |  2 >5% |  3 >25% |  4 >50% |
|  | 7.1 Path width (m) |  0 <1 |  1 <2 |  2 <3 |  3 <4 |  4 <5 |
|  | 7.2 Paths incised as % of the frontal dune height |  0 <5% |  1 <25% |  2 <50% |  3 >50% |  4 <75% |
|  | 8. Anthropogenic litter: cover as % surface |  0 0% |  1 <5% |  2 >5% |  3 >25% |  4 >50% |
|  | 9. Amount of sand (%) extracted for construction, etc. |  0 0% |  1 <5% |  2 >5% |  3 >25% |  4 >50% |
|  | 10. Summer beach cleaning frequency. (High is twice a day; medium is daily) |  0 Low |  |  2 Moderate |  |  4 High |
|  | 11. % upper beach cleaned |  0 0% |  1 <25% |  2 <50% |  3 <75% |  4 >75% |
|  | 12. % permanent infrastructure replacing active dunes (roads, houses, etc.) |  0 0% |  1 <25% |  2 <50% |  3 <75% |  4 >75% |
|  | 13. % ephemeral infrastructure replacing active dunes (outdoor facilities, camping) |  0 0% |  1 <25% |  2 <50% |  3 <75% |  4 >75% |
|  | 14. Relative surface (%) forested in the system (200 m inland from the foredune) |  0 0% |  1 <25% |  2 <50% |  3 <75% |  4 >75% |
|  | 15. Relative cover (%) of agriculture in the system (200 m inland from the foredune) |  0 0% |  1 <25% |  2 <50% |  3 <75% |  4 >75% |
|  | 16. Grazing on the active system |  0 None |  |  2 Moderate |  |  4 Intensive |
|  | 17. Rabbit numbers |  0 None |  1 Low |  2 Moderate |  3 High |  4 Intensive |
|  | Total score/per cent |  |  |  |  |  |

References

García-Mora, M.R., Gallego-Fernández, J.B., Williams, A.T. & García-Novo, F. (2001) A coastal dune vulnerability classification. A case study of the SW Iberian Peninsula. Journal of Coastal Research 17: 802–811.

Williams, A.T., Alveirinho-Dias, J, García-Novo, F., García-Mora, M.R., Curr, R. & Pereira, A. (2001) Integrated coastal dune management: checklists. Continental Shelf Research 21:1937–1960.