Paying the forest for electricity: a modelling framework to market forest conservation as payment for ecosystem services benefiting hydropower generation

MAURICIO E. ARIAS1, THOMAS A. COCHRANE1*, KEITH LAWRENCE2, TIMOTHY J. KILLEEN2 AND TRACY A. FARRELL2

APPENDIX 1

Terrain, soils, precipitation and land use data from the Pursat 1 catchment used for predicting sediment yields with the SWAT model. Elevations are in metres above mean sea level, area estimates in hectares and rainfall in millimetres. Data from Aruna Technology Ltd (2006) and MRC (2009b).

| Terrain | Elevation range (m) | Elevation mean (m) | Elevation std dev | Area slope 0–5% (ha) | Area slope 5–20% (ha) | Area slope 20–345% (ha) | Slope median |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 155–1700 | 406 | 267 | 50763 | 30188 | 30423 | 15% |
| Soils | Class |  | Area | % clay | % silt | % sand | Hydrologic group |
|  | Red-yellow podzols |  | 35863 | 3 | 16 | 81 | A |
|  | Acid lithosols |  | 61555 | 18 | 39 | 43 | D |
| Rainfall | Min annual (mm) | Max annual (mm) | Mean annual (mm) | Min month (mm) | Max month (mm) |  |  |
|  | 1179 | 1580 | 1380 | 0 (Jan) | 454 (Oct) |  |  |
| Land use (in 2006) | Class |  | Area (ha) | % area |  |  |  |
|  | Agricultural |  | 3014 | 2.7 |  |  |  |
|  | Deciduous forest |  | 26976 | 24.2 |  |  |  |
|  | Evergreen forest |  | 67141 | 60.3 |  |  |  |
|  | Mixed forest |  | 415 | 0.4 |  |  |  |
|  | Reservoir |  | 13830 | 12.4 |  |  |  |

Figure 1A Predicted fraction of original reservoir storage for the forest conservation and deforestation scenario.