SUPPLEMENTARY MATERIAL

Table S1- Distribution system pipe condition categories as outlined in the Canadian Infrastructure Report Card (2013)

| Physical Condition | Distribution System |
| --- | --- |
| 5 – Excellent | - No structural defects. - Little or no water loss through leakage. |
| 4 – Good | - Minor cracking, spalling or signs of wear. - Deterioration causing minimal influences on levels of service - Less than 1 break/km/year. - Equivalent to OFWAT condition grade 2. |
| 3 – Fair | - Medium cracking, spalling or signs of wear. - Deterioration beginning to be reflected in deteriorating levels of service and/or increased operating costs. - Less than 3 breaks/km/year. - Equivalent to OFWAT condition grade 3. |
| 2 – Poor | - Fracture with deformation up to 10%. - Nearing the end of useful life, further deterioration likely, affecting levels of service. - Greater than or equal to 3-5 breaks/km/year. - Equivalent to OFWAT condition grade 4. |
| 1 – Very Poor | - Collapsed or collapse imminent. - No residual life expectancy, requires urgent replacement. - Equivalent to OFWAT condition grade 5. |

Table S2. Summary of selected RCTs, corresponding ranking scores and percentiles used to rank the systems among Canadian municipal drinking water systems

| Author, publication date | Study area | Source water | Treatment | Treatment Category | Source water quality (cfu E. coli/ 100mL) | Population Served | Ranking Score | Percentile rank among Canadian systems |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Payment et al. (1991) | Laval, Canada | River water | Flocculation, Rapid sand filtration, ozonation, chemical disinfection | 1 | 367 cfu/ 100mL | >10,000 | 41.23 | 0.169 |
| Payment et al. (1997) | Laval, Canada | River water | Flocculation, Rapid sand filtration, ozonation, chemical disinfection | 1 | 367 cfu/ 100mL | >10,000 | 41.23 | 0.169 |
| Hellard et al. (2001) | Melbourne, Australia | Surface water source from protected catchment | Chlorination | 9 | 0 cfu/ 100mL | >10,000 | 53.33 | 0.430 |
| Colford et al. (2005) | Davenport, USA | River water | Conventional treatment with granular activated carbon/ sand filters & chlorination | 1 | Fecal coliforms 577 cfu/ 100mL | >10,000 | 41.23 | 0.169 |

1The AGI rates published in the various randomized controlled trials were documented in terms of highly credible gastrointestinal illness (HCGI) which is a less restrictive definition of AGI. These HCGI rates were converted to AGI rates using estimated total Canadian HCGI and AGI rates of 1.3 cases/ person-year and 0.63 cases/ person-year, respectively (Thomas et al. 2013). Reported rates from the RCTs were multiplied by 0.63/ 1.3.

Table S3.  Summary of the categories resulting from the ranking of RCTs among Canadian drinking water systems and distribution systems including percentile ranking, population served, AGI incidence rates (cases/person-year) assigned for distribution system and source water/treatment risk

| Category | Groupings of Canadian Systems | Percentile groupings for treatment system Ranking | Corresponding RCTs that fall in the category | Corresponding distribution system groupings, by percentile | Population Served | SW/TR AGI distribution assigned4 | Distribution system AGI Risk4 | Fraction of AGI due to Distribution4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Systems that are ≤ 16.9%ile1 SW/TR2; ≤9.3%ile DS3 | 0 to 16.9 | Payment et al. 1991 Payment et al. 1997 Colford et al. 2005 | 0- 0.093 (13.8% population) | 1, 371,086 | Pert (0, 0.0388, 0.126)1 | 0.01-0.03 | Uniform (0- 0.03/ 0.0388) 0- 77% |
| 2 | Systems that are 17.0-42.9%ile for SW/TR and ≤9.3%ile DS | 17.0 to 42.9 | Assumption: bordered by Payment et al. 1997 and Hellard et al. 2001 | 0- 0.093 (13.8% population) | 2,564,917 | Pert (0, 0.0145, 0.0388)2 | 0.01-0.03 | Uniform (0- 0.03/ 0.0145) 0- 100% |
| 3 | Systems that are 17.0-42.9%ile for SW/TR and ≥9.4%ile DS | 17.0 to 42.9 | Assmption: bordered by Payment et al. 1997 and Hellard et al. 2001 | 0.094- 1.00 (86.2% population) | 16,385,504 | Pert (0, 0.0145, 0.0388)2 | 0-0.01 | Uniform (0- 0.01/ 0.0145) 0-69% |
| 4 | Systems that are ≥43%ile for SW/TR and ≥9.4%ile DS | 43.0- 100 | Hellard et al. 2001 | 0.094-1.00 (86.2% population) | 8,200,256 | Pert (0, 0.00725,0.0145)3 | 0-0.01 | Uniform (0- 0.01/ 0.0145) 0-69% |

1%ile= Percentile

2SW/TR = source water/treatment

3DS=distribution system

4AGI attributable risk units= cases/person-year

Table S4. Summary of calculations performed and key model inputs used in the determination of AGI attributable to municipal drinking water systems serving greater than 1000 people in Canada

| Category | Total Population (Column A) | Population (excluding bottled water) (Column B= Column A-22%) | Fraction DS1 (Column C) | Fraction AGI SW/TR2 (Column D) | AGI Total (Column E)3 | AGI SW/TR (Column F) | AGI DS (Column G) | Cases SW/TR (Column H) | Cases DS (Column I) | Total Cases |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1,371,086 | 1,069,447 | 0-77% | 1- Column C | Pert( 0, 0.0388, 0.126) | D*E | C*E | B*F | B * G | Sum (H +I) |
| 2 | 2,564,917 | 2,000,635 | 0-100% | 1- Column C | Pert (0, 0.0145, 0.0388) | D*E | C*E | B*F | B * G | Sum (H+I) |
| 3 | 16,385,504 | 12,780,693 | 0-69% | 1- Column C | Pert (0, 0.0145, 0.0388) | D*E | C*E | B*F | B * G | Sum (H+I) |
| 4 | 8,200,256 | 6,396,200 | 0-69% | 1- Column C | Pert(0, 0.00725,0.0145) | D*E | C*E | B*F | B * G | Sum (H+I) |
| Total |  |  |  |  |  | = Sum Group 1-4 (Columns H+I) |  |  |  |  |

1DS- distribution system

2SW/TR – source water/ treatment

3AGI attributable risk units= cases/ person-year