Supplementary document

2-kilowatt-level superfluorescent fiber source with flexible wavelength and linewidth tunable characteristics

Jun Ye1, Chenchen Fan1, Jiangming Xu1, Hu Xiao1, Jinyong Leng1, and Pu Zhou1

1 College of Advanced Interdisciplinary Studies, National University of Defense Technology, Changsha 410073, China

The parameter values used for numerical calculation are listed in Table S1 and Table S2. Note that the well-known physical constants such as the Planck constant h and the Boltzmann constant kB are not provided, and the absorption and emission cross-sections of ytterbium-doped fiber (YDF) dependence on the wavelength can be obtained from the supplementary Ref. [1] as well as the fiber supplier. As for the simulation method of the combining of rate equations and generalized nonlinear Schrödinger equation (GNLSE), here we utilized the well-known split-step Fourier method (SSFM), one can refer to supplementary Refs. [2, 3], and a basic computer code can also be found in the famous book Nonlinear Fiber Optics written by Prof. G. P. Agrawal (Pages 615–617, 5th edition, Academic Press, 2013).

Table S1. Parameters for the numerical calculations (10/125 YDF).

| Parameter | Symbol | Value |
| --- | --- | --- |
| Loss coefficients | αp, αs | 3, 5×10-3 m-1 |
| Dispersion coefficients | β2, β3 | 15.6 ps2/km, 0.05 ps3/km |
| Nonlinear Kerr coefficient | γ | 1.8 W-1/km |
| ytterbium dopant concentration | N0 | 6.23×1025 m-3 |
| Overlap factors | Γp, Γs | 0.0064, 0.85 |
| Lifespan of excited state population | τ | 840 μs |
| Temperature | T | 298 K |

Table S2. Parameters for the numerical calculations (20/400 YDF).

| Parameter | Symbol | Value |
| --- | --- | --- |
| Loss coefficients | αp, αs | 3, 5×10-3 m-1 |
| Dispersion coefficients | β2, β3 | 20.4 ps2/km, 0.04 ps3/km |
| Nonlinear Kerr coefficient | γ | 0.5 W-1/km |
| ytterbium dopant concentration | N0 | 7.8×1025 m-3 |
| Overlap factors | Γp, Γs | 0.0025, 0.85 |
| Lifespan of excited state population | τ | 840 μs |
| Temperature | T | 298 K |

Supplementary references

[1] Paschotta, R., Nilsson, J., Tropper, A. C., and Hanna, D. C., “Ytterbium-doped fiber amplifiers”, IEEE J. Quantum Elect. 33, 1049–1056 (1997).

[2] Liu, W., Ma, P., Lv, H., Xu, J., Zhou, P., and Jiang, Z., “Investigation of stimulated Raman scattering effect in high-power fiber amplifiers seeded by narrow-band filtered superfluorescent source”, Opt. Express 24, 8708–8717 (2016).

[3] Turitsyn, S. K., Bednyakova, A. E., Fedoruk, M. P., Latkin, A. I., Fotiadi, A. A., Kurkov, A. S., and Sholokhov, E., “Modeling of CW Yb-doped fiber lasers with highly nonlinear cavity dynamics”, Opt. Express 19, 8394–8405 (2011).

___________________________________________________________________________

Correspondence to: Jiangming Xu and Pu Zhou, College of Advanced Interdisciplinary Studies, National University of Defense Technology, Changsha 410073, China. Email: jmxu1988@163.com (J. Xu), zhoupu203@163.com (P. Zhou).